1502 Vine Street, 3rd Floor  Cincinnati, Ohio 45202

March 6, 2026

Via Email:  Lisa May <Lisa.May@mdrealtor.org>

Ms. Lisa May

Maryland Association of Realtors

RE:  Salisbury Accessory Dwelling Unit Ordinance

Dear Ms. May,

On behalf of the Maryland Association of Realtors, ZoneCo has reviewed the ordinances pertaining to Accessory Dwelling Units in Salisbury, Maryland.  Salisbury Zoning regulations, Chapter 17.164, currently permit the construction of an attached apartment, or detached accessory dwelling, on any property with a single-family detached dwelling in the R-5S, R-8S, and R-10S zoning districts, under the umbrella of special exception accessory structures.  To be eligible for an accessory apartment, the lot must be a minimum of 60 feet wide and contain 9,000 square feet of land area. Accessory dwellings are not limited to occupancy by familial relationship, such as in-law apartments. Property owners are required to reside in either the primary dwelling unit or in the accessory dwelling on the property. Certain size and setback requirements must be met. A building permit is required.  The Accessory Apartment ordinance has not been revised since the adoption of HB 1466.

Our review considers (A) how the ordinance complies with Maryland HB 1466 adopted in 2025, and (B) how the ordinance compares with recommended provisions by MD Realtors.  We have also attached comments with the proposed Salisbury Ordinance.

(A) Comparison with required provisions Maryland HB 1466 adopted in 2025.

1. 	Allowed By Right:

- State Law Requires:  The state law requires that Accessory Dwellings be permitted by right within any zone that permits single-unit detached residential development.

- Salisbury Ordinance:  DOES NOT COMPLY –

  - The Salisbury Ordinance allows “accessory apartments” as special exception accessory structures for all detached dwellings within the R-5S, R-8S, and R-10S zoning districts. The Special Exception process requires a public hearing and is not a by-right permission.  To comply with the state law, permission for an accessory apartment must be by-right, (not requiring a public hearing), so long as reasonable standards are met.

  - The Salisbury Ordinance has additional lot criteria, that to be eligible for an accessory apartment, the lot must be a minimum of 60 feet wide and contain 9,000 square feet of land area.  This additional criteria should be removed, as the state law requires by right approval in any zone that allows single unit detached residential development.

- No Density or Growth Restrictions:

  - State Law Requires:  A local jurisdiction may not regulate or restrict ADU’s using residential density restrictions or residential growth limitation measures.

  - Salisbury Ordinance:   COMPLIES - The Ordinance treats ADU’s as special exemption accessory structures with no APFO requirements.

- Must Comply with Safety & Building Codes:

  - State Law Requires:  Must provide for construction of ADUs that meet public health, safety, and welfare standards, including relevant building codes.

  - Salisbury Ordinance:  COMPLIES - The existing ordinance does not exempt accessory apartments from applicable building construction and Maryland Department of Health and Mental Hygiene and MDE Codes.

## 4. 	Setbacks Same As Accessory Use Setbacks

- State Law Requires:  May not require side or rear yard setbacks that are greater than the side and rear yard setbacks for accessory structures in the applicable zoning district.  Also note:  In determining setbacks for detached ADUs, many jurisdictions have used five feet as the minimum separation between the ADU structure and the property line so that the accessory dwelling structure does not have to meet building code standard fire resistance rating requirements. Also, a five-foot setback option allows for more windows and flexibility in window placement.

- Salisbury Proposed:   COMPLIES:  The setback requirements for detached accessory apartments are the same as “Accessory Structures”,  with the law.

## 5. 	ADU Definition

- State Law Requires:  The bill defines an ADU as “a secondary dwelling unit that is:

  - on the same lot, parcel, or tract as a primary single-family detached dwelling unit and

  - not greater than 75% of the size of and subordinate in use to the primary single-family detached dwelling unit.”

- Salisbury Ordinance:  DOES NOT COMPLY –

  - "Accessory apartment" are defined in the Salisbury Ordinance as “a dwelling unit, limited to no more than one bedroom, incorporated within a single-family dwelling or its accessory building. The accessory apartment or the principal dwelling shall be occupied by the owner(s) of the property on which the accessory apartment is located.”

  - Accessory Apartments are allowed in some Residential Districts as an accessory use, but not necessarily on all parcels with a primary single-family detached dwelling unit.  Recommendation:  The language in the Salisbury Ordinance should track the state law definition of an ADU.

  - The size of both attached and detached accessory apartments is limited not by definition, but in §17.164.030 An accessory apartment is limited to 500 gross square feet in size.   This is a very small apartment size.

It is unclear if the state law requires jurisdictions to permit the accessory apartment to have “up to 75% of the primary dwelling unit floor area” or allows a local jurisdiction to permit a floor area maximum that is less than 75% of the primary dwelling unit (for example up to 50% of the primary dwelling unit floor area).

Recommendation: Experienced accessory dwelling unit builders have indicated that 900 square feet is a minimum floor area to allow flexibility for a second bedroom, which is commonly requested for use as a guest bedroom, a home office, storage, a hobby or craft room, or similar ancillaary use.  Some jurisdictions have also allowed an exception to the maximum floor area when the accessory apartment occupies a basement or attic space that has the same floor area as the main floor footprint.  We recommend that the ordinance be revised to either match the maximum floor area of the state law, or at a minimum, permit an accessory apartment up to 900 square feet.

(B) Comparison of Salisbury Ordinance with recommended ADU ordinance elements.

## 1. 	Owner Occupancy

- Recommendation:  Owner-occupancy should not required for either the primary dwelling or the accessory dwelling.

i. Jurisdictional approaches on this can vary depending on whether short-term rentals are permitted.  Where short-term rental is allowed for the primary or accessory dwelling, it is common to require owneroccupancy of the other dwelling.  In their recently adopted ordinance, Washington County decided not to require owneroccupancy to encourage development of accessory dwellings to address the housing crisis.  This means that both the primary dwelling unit and the accessory apartment could be rented.  However, Washington County does not permit short-term rental of either unit, as the goal of their accessory dwelling ordinance was to increase the number of dwellings.

- Salisbury Ordinance:   DOES NOT COMPLY.  The owner must reside in either the single-family dwelling or the ADU.  Recommend revising so that owner-occupancy is not required for either unit, per the above recommendation.

## 2. 	Parking

- Recommendation:  Because of common space constraints and the additional cost, additional parking should not be required if the principal dwelling unit has sufficient off-street parking, or adequate additional onstreet parking is available within walking distance (500 feet) of the lot containing the accessory dwelling.  Also recommend staff-level authority to approve parking sufficiency.

- Salisbury Ordinance:  § 17.196.030 implies that 2 off-street parking spaces are required, as an accessory apartment is a “residential use”.  Revise to clarify the parking requirements and comply with above recommendation.

## 3. 	Minimum Floor Area

- Recommendation:  No minimum floor area should be required.  If a minimum floor area is required, it should be no less than 400 sq. ft. to permit a studio apartment.

- Salisbury Ordinance: No minimum floor area specified.   No change recommended.

## 4. 	Maximum Floor Area

- Required per State Law:  The ADU floor area should not be greater than 75% of the gross floor area of the primary dwelling unit.

- Salisbury Ordinance:  The maximum floor area for both attached and detached accessory apartments is limited to 500 gross square feet.  Recommend changing to match recommendation in A.5.b.3 above.

## 5. 	# Bedrooms

- Recommendation:  No more than 2 bedrooms are permitted in an ADU.  Limiting to no more than 2 bedrooms is appropriate given the recommendation to waive other requirements, such as off-street parking, impact fees, APFO compliance, and separate water and sewer taps.

- Salisbury Ordinance:  Accessory apartments are limited to one bedroom by definition.  This is a fairly strict requirement, and could limit the number of accessory apartments constructed. For example, accessory apartments that provide housing for seniors benefit from having a second bedroom so that a caregiver can sleep over. Revise per the above recommendation to permit up to 2 bedrooms.

6. 	Definitions for ADU Types (for example: Attached ADU, Detached ADU,

Internal ADU)

- Recommendation:  These definitions are optional.  Definitions may or may not be needed depending on other criteria imposed.  For example, if setback requirements vary depending on whether an accessory dwelling is attached or detached, then definitions are needed.

- Salisbury Ordinance:  Salisbury provides one definition that encompasses both attached and detached accessory apartments.  No change recommended.

## 7. 	Approval Process

- Recommendation:  Strongly recommend that an ADU that meets all zoning criteria can be approved by staff without a public hearing.

- Salisbury Ordinance: Accessory apartments require a special exception.  Revise per A.1.b above, to allow administrative review and approval Accessory apartment that meets the requirements.

8. 	Vehicular Access (for example, access off public street vs. alley)

- Recommendation:  Recommend not specifying the vehicular access for the accessory dwelling, and allow staff-level review and approval of adequate vehicular access.  The State law allows, but does not require, a local jurisdiction to prohibit conversion of an accessory structure to an accessory dwelling if the only vehicular access is an alley.

- Salisbury Ordinance: The type of vehicular access is not specified.  No change recommended.

## 9. 	Front Door Access

- Recommendation:  An ADU should have a direct exterior access that is separate from the existing primary dwelling.  Recommend that administrator has discretion to waive this requirement if the entrance does not go into or through the primary dwelling unit

- Salisbury Ordinance:  Exterior access is not mentioned.  Recommend adding a provision regarding Front Door Access.

## 10. 	Non-conforming lots

- Recommendation:  To allow flexibility for an accessory dwelling on an existing lot of record with a non-conforming lot size and/or a nonconforming setback, allow an accessory dwelling so long as the ADU does not increase the non-conforming condition.

- Salisbury Ordinance:  Because accessory apartments are classified as an accessory structure, a non-conforming lot size would not prevent development of an accessory apartment so long as the required lot size and setbacks are provided. No change recommended.

## 11. 	Short-Term Rentals

- Recommendation:  No short-term rental restrictions

- Salisbury Ordinance:   Salisbury does not prohibit short-term rental of ADU’s. The city does require operators of short-term rentals to obtain a rental license.  No change recommended.

## 12. 	APFO & Impact Fees

- Recommendation:  An ADU should not have to comply with the APFO Ordinance.  Due to their small size, ADU’s have not had noticeable impacts on public facilities.

- Salisbury Ordinance:   An ADU is considered an accessory structure and does not have to comply with APFO and Impact Fee requirements.  No change recommended.

(C) Building Code Issues.  Most construction standards and utility requirements, such as fire sprinkler systems, and water and sewer service, are typically addressed in each jurisdiction’s building or health and safety codes.  These issues are typically not regulated in zoning code ordinances.  In some cases, the construction standards are mandated by state law.  You should be aware that certain building code requirements can add significantly to the cost of constructing an ADU, to the point that adding an ADU may become cost-prohibitive for some homeowners.  The following issues can have significant impact on the cost of constructing an ADU.

- Fire Sprinkler Systems:  By current State law, ADU’s are generally not required to have sprinkler systems if they are created within existing dwellings that do not have sprinkler systems.  However, Maryland requires sprinkler systems to be installed in all

newly constructed single-family and two-family dwellings, including ADU’s.

Maryland Building Performance Standards Regulations, Md. Code Ann. §9.12.51.05(B)(1)(d).

- Well & Septic:  To encourage ADU development, an ADU should be permitted to connect to the well and septic system for the primary dwelling unit if adequate capacity exists.  Typically well and septic systems are regulated by the local health department.  If local health departments require a separate system, we recommend that local jurisdictions should change their policy to allow joint use of existing sell and septic systems where physically possible.

- Public Water & Sewer:  To encourage ADU development, wherever physically possible, an ADU should be permitted to be serviced by the same public water and sewer lines as the principal dwelling unit to unnecessary expense including additional tap fees, which can be very expensive.

(C) Summary recommendations for an ADU Ordinance for Salisbury.  Many provisions of the existing accessory apartment provisions comply with the new state law and best practices.  A few aspects of the Salisbury accessory apartment ordinance should be revised as follows:

- Maximum Size:  The maximum size of an accessory apartment, whether attached or detached, should be revised to comply with the state law, that an accessory apartment can be “no greater than 75% of the size of and subordinate in use to the primary single-family detached dwelling unit;” or alternatively, allow a maximum size of at least 900 -1,200 sq. ft.; or for apartments that occupy one floor within a primary dwelling (such as a basement or attic apartment), up to the size of the main floor footprint.

- Lot size:  The requirement that accessory apartments are only permitted on lots that have minimum of 60 feet wide and contain 9,000 square feet of land area, should be removed, as the state law requires by right approval in any zone that allows single unit detached residential development.

- Owner Occupancy:  Ideally, owner-occupancy should not be required for either the primary dwelling or the accessory dwelling for maximum flexibility.  However, this may not be politically possible currently, and the policy could be revisited at a later date.

- Bedrooms: A maximum of 2 bedrooms should be permitted.

- Parking: Additional parking should not be required if the principal dwelling has sufficient off-street parking, or adequate additional on-street parking is available within walking distance (500 feet) of the lot containing the accessory dwelling.  Also recommend staff-level authority to approve parking sufficiency.

Thank you for the opportunity to review the local ordinance.  Please feel free to reach out if you have any questions.

Sincerely, Teresa Bamberger

Teresa Bamberger

Encl:   Salisbury “Accessory Apartment” Ordinance

ADU Guide for Local Jurisdictions

### Title 17 - ZONING Chapter 17.164 R-5S, R-8S AND R-10S RESIDENTIAL DISTRICTS

Chapter 17.164 R-5S, R-8S AND R-10S RESIDENTIAL DISTRICTS

17.164.010 Purpose.

The purpose of the R-5S, R-8S and R-10S residential districts is to preserve the existing character of both newly planned and existing single-family residential areas in order to promote and to enhance the quality of life and environmental attributes which are essential parts of the city. The uses permitted in these districts are limited primarily to single-family residential, with selected nonresidential uses which provide a service to the residents of an area or which, by their nature, require a residential environment. Multifamily developments, therefore, are incompatible because they generate an undue concentration of population and increased traffic which alter the predominantly single-family residential character of these areas. The following uses, standards and area regulations were developed based upon this purpose.

(Prior code § 150-20.1)

17.164.020 Permitted uses.

Permitted uses shall be as follows:

- Cluster development of single-family detached dwellings or patio dwellings in accordance with chapter

17.176.

- Cultivation of land;

- Dwellings.

  - Patio, in accordance with chapter 17.200;

  - Single-family detached;

- Firehouse;

- Park and playground, public and private, in accordance with chapter 17.220;

- School of general instruction, in accordance with chapter 17.220.

(Prior code § 150-20.2)

17.164.030 Uses permitted by special exception.

Uses permitted by special exception shall be as follows:

- An accessory apartment of not over five hundred (500) gross square feet in size within an owneroccupied single-family detached dwelling or in an accessory building on a lot upon which an owneroccupied single-family detached dwelling is located. The lot shall have a minimum sixty (60) feet width and contain at least nine thousand (9,000) square feet of land area. All parking required for an accessory apartment shall be in the rear yard no closer than three (3) feet to all adjoining property lines, provided that an accessory apartment shall not be allowed in any dwelling where guest rooms are provided;

- Care home, in accordance with chapter 17.220;

Salisbury, Maryland, Municipal Code

- Cemetery, on a minimum tract of ten acres;

- Church and other place of worship on a lot size of less than five acres, in accordance with chapter

17.220, excluding bus storage, cemetery, day-care center, school of general instruction and gymnasium as accessory uses;

- Church and other place of worship on a minimum lot of five acres, in accordance with chapter 17.220, including an activity building with offices and meeting rooms, cemetery, day-care center, school of general instruction and gymnasium as accessory uses;

- Day-care facilities for the elderly and handicapped;

- Group domiciliary care facility;

- Rental of guest rooms to not more than two roomers in an owner-occupied single-family detached dwelling on a minimum lot of five thousand (5,000) square feet, provided that one additional parking space shall be provided in the rear yard for each roomer, and provided further that the rental of guest rooms shall not be allowed in any dwelling or on any lot that contains an accessory apartment.

(Prior code § 150-20.3)

17.164.040 Uses permitted by ordinance permit.

Uses permitted by ordinance permit by the city council shall be as follows:

- Day-care center or nursery school, in accordance with chapter 17.220;

- Utility substation, in accordance with chapter 17.220.

(Prior code § 150-20.4)

17.164.050 Accessory uses and structures.

Accessory uses and structures shall be as follows:

- Cloister or clerical housing in the same lot with a church or other place of worship, meeting lot area and/or standards required for each individual use;

- Home occupation;

- Home office;

- Family day-care home;

- Private garages and other accessory uses normally associated with a residential use, such as but not limited to detached home workshop, swimming pool, cabana, greenhouse, private studio and boathouse, all of which shall be incidental to the use of the property as a residence;

- Storage of recreational vehicles and boats on residential lots, limited to two in any combination, in back of the front building setback line, where such recreation vehicles and boats are for the use and enjoyment of the resident thereon;

- Other accessory uses and structures clearly incidental to, customary to and associated with the permitted use.

(Prior code § 150-20.5)

17.164.060 Development standards.

Development standards for the R-5S, R-8S and R-10S residential districts shall be as follows:

A. 	Minimum Lot Requirements. All lots hereafter established shall meet the following minimum requirements:

District  	Lot Area  	Interior Lot Width  	Corner Lot Width

(square feet)  	(feet)  	(feet)

### R-5S  	5,000  	50  	65  R-8S  	8,000  	60  	75

R-10S  	10,000  	70  	85

B. 	Minimum yard and setback requirements shall be as follows:

- Front: twenty-five (25) feet;

- Rear: thirty (30) feet;

- Side: ten feet each; two required.  C. Height Limitations.

- The height limitation for principal buildings and structures shall be thirty-five (35) feet.

- The height limitation for accessory buildings and structures shall not exceed twenty (20) feet.  D. 	Parking shall be provided in accordance with chapter 17.196.

- The parking of vehicles of any type shall not be permitted within the front yard other than driveways providing access from public streets.

- No outside storage of trucks or vans used in the conduct of business shall be permitted.  E. 	No more than one principal use shall be permitted on an individual lot.

- Accessory Buildings and Structures.

  - No part of any accessory building or structure shall be located closer than five feet to a rear or side property line. On a corner lot, no accessory building shall be located closer than twenty-five (25) feet to the curbline of an abutting street.

  - No accessory building or structure shall occupy more than fifty (50) percent of the required rear yard or side yard area.

  - Swimming pools may be constructed in the rear yard or in a side or front yard on a corner lot, no closer than twenty-five (25) feet to any curbline or property line if no curbline exists; provided, that the combined total coverage of a swimming pool and all accessory buildings or structures, including those allowed to project into yards, shall not occupy more than seventy-five (75) percent of the required rear or side yard.

- Signs. All signs shall be in accordance with the provisions of chapter 17.216 for an R-5 district.

- Landscaping or Screening.

  - Landscaping or screening shall be provided for all uses in accordance with the provisions of chapter 17.220.

  - In addition to the requirements of chapter 17.220, all areas not devoted to building or required parking areas shall be landscaped as defined in Section 17.04.120 and maintained in accordance with Section 17.220.080.  I. 	Related Requirements.

  - The provisions of chapter 17.04, Article IV, where applicable, shall apply to all uses and structures relative to vision at intersections, height exceptions, yard exceptions, fences and walls, airport height limitations and historic or religious monuments, markers or shrines.

  - Projections into yards may be allowed in accordance with the provisions of chapter 17.04, Section 17.04.230.

(Ord. 1599 § 16 (part), 1995; prior code § 150-20.6)

## ACCESSORY DWELLING UNITS

A Guide for Local Implementation of  Maryland 2025 HB 1466, SB 891

Table of Contents:

Summary of Maryland Accessory Dwelling Legislation	2

Recommended Model ADU Ordinance	4

ADU Ordinance - Required Provisions	7

ADU Ordinance - Optional Provisions	8

Examples of Existing ADU Ordinances in Maryland ........................... 12

ADU Ordinance – Other Resources	15

Sponsored by:

# Summary of Maryland Accessory Dwelling Legislation

PURPOSE:  The 2025 legislation seeks to address Maryland’s estimated 96,000 housing unit shortage and housing affordability crisis by requiring local jurisdictions to adopt local laws allowing accessory dwellings.  (Ref:  Maryland House Bill 1466/Senate Bill 891, Effective October 1, 2025.)

ADOPTION DATE:  Each legislative body of a local jurisdiction (county or municipality) must adopt – by October 1, 2026 – a local law authorizing the development of accessory dwellings (“ADU’s”) in accordance with the bill.

The bill defines an ADU as:

“A secondary dwelling unit that is:

- on the same lot, parcel, or tract as a primary single-family detached dwelling unit and

- not greater than 75% of the size of and subordinate in use to the primary single-family detached dwelling unit.”

An ADU includes a structure that is within, separate from, or attached as an addition to, the primary detached dwelling structure.

ACCESSORY DWELLING UNIT PROVISIONS TO BE ADDRESSED BY LOCAL ZONING CODES:

A local law adopted under the bill MUST:

- provide for construction of ADUs that meet public health, safety, and welfare standards, including relevant building codes and adequate public facilities provisions;

- exclude the development of an ADU from the calculation of density and application of any residential growth limitation measures; and

- not establish setback requirements that exceed the existing accessory structure setback requirements from the side and rear lot lines.

A local law adopted under the bill MAY:

- establish standards for ADU safety;

- prohibit the full or partial conversion of an accessory structure as an ADU if the only vehicular access to the accessory structure is from an alley; and

- establish additional off-street parking requirements (provided a parking study is completed and a waiver process is established) that consider, among other things, the cost to construct off-street parking, and whether sufficient curb area exists along the front line of the property to accommodate on-street parking.

Restrictions on Use: A restriction on use (for example HOA bylaws or rules, a condition in a deed, a contract, or a security instrument) may not impose or act to impose an unreasonable limitation on the ability of the property owner to develop or offer for rent an ADU.

An unreasonable limitation:

- includes a limitation that prohibits, either explicitly or by effect of the restrictions, the development of an ADU and

- does not include a limitation on the short-term rental of an ADU.

Homeowners Association Voting and Assessments: The bill authorizes the governing body of an HOA to treat an ADU as a separate lot for purposes of voting on HOA matters and levying assessments.

# Recommended Model ADU Ordinance

ORDINANCE NO. ___

AN ORDINANCE OF  	 	(name of jurisdiction) 	 	TO ALLOW

AN ACCESSORY DWELLING ON LOT OF RECORD IN ANY ZONE THAT ALLOWS A DETACHED SINGLE-UNIT DWELLING

WHEREAS, our community is experiencing a lack of housing, resulting in families and households of middle and moderate incomes finding housing prices and rents to be unaffordable; and

WHEREAS, our community has also determined that the housing crisis is creating difficulties for employers hoping to attract new employees to our community; and

WHEREAS, our community has also determined that the housing crisis is impacting the wellbeing of our residents, particularly lower-income and middle-income earners; and

WHEREAS, accessory dwellings offer a way to provide compact, relatively affordable housing in established neighborhoods with minimal impact to infrastructure and to supply new housing opportunities without added dispersed low-density housing; and

WHEREAS, accessory dwelling units provide families with options for intergenerational living arrangements that enable child or elder care and aging in place, and accessory dwelling units enable seniors to downsize, move into accessible units, or live with family or a caregiver while remaining in their communities; and

WHEREAS, accessory dwelling units can provide rental income to help a variety of residents, such as older homeowners on fixed incomes, first-time homebuyers, and low- and moderate-income homeowners; and

WHEREAS, our community can benefit from the creation of accessory dwellings on residential properties, which can provide employee housing, more affordable housing, housing for the aging, housing for young adults, and housing for multi-generational households on the same lot.

NOW, THEREFORE, based upon the foregoing, be it hereby ordered by the

(town council/county commissioners) 	 	 	 	 that the Zoning Code for

(name of jurisdiction) 	 	 	 be amended to allow Accessory

Dwellings as follows:

| (Insert Zoning Code Section/Chapter Reference Number) Definitions: Accessory Dwelling. A second dwelling that is: (1) located on the same lot, parcel, or tract as a primary dwelling, and (2) has a gross floor area not greater than 75% of the size of the primary dwelling, and (3) is subordinate in use to the primary dwelling. Primary Dwelling. A single-unit detached dwelling that is the principal structure on a single lot of record, and is the larger of the dwellings if there is an accessory dwelling located on the same lot. (Insert Zoning Code Section/Chapter Reference Number) “Accessory dwelling.” An accessory dwelling is a permitted use in any zone that permits a primary dwelling as a principal use, subject to the following conditions: - The Zoning Administrator shall review all applications for an accessory dwelling and may approve such applications upon finding that the proposed accessory dwelling substantially complies with these conditions. - A maximum of one accessory dwelling may be established on any lot with a primary dwelling as a principal use. - The floor area of any accessory dwelling may be no more than 75% of the gross floor area of the principal dwelling. The calculation of the accessory dwelling floor area shall not include loft or attic space used solely for storage. D. An accessory dwelling shall contain no more than 2 bedrooms. - An accessory dwelling shall not count as a dwelling unit for purposes of calculating housing density. - An accessory dwelling shall meet public health, safety, and welfare standards, including relevant building codes and adequate public facilities provisions. - An accessory dwelling is not required to comply with Adequate Public Facilities Ordinance requirements related to schools. |  |
| --- | --- |

| - An accessory dwelling may be established within a principal dwelling or within an accessory structure, and shall maintain the following setbacks: - An accessory dwelling located within a principal dwelling, or attached to a principal dwelling along a common wall, must comply with building setbacks applicable to the principal dwelling. - An accessory dwelling that is not attached to the principal dwelling must comply with applicable accessory structure setbacks of the zoning district, however, in no case shall an accessory dwelling be setback from any property line less than 5 feet. - Mechanical equipment for an accessory dwelling may project into the required building setback. - Each accessory dwelling shall have its own separate exterior door providing ingress and egress. - An accessory dwelling must have the same street address (house number) as the primary dwelling. - Both the accessory dwelling and the principal dwelling shall be located on one lot in common ownership. - An accessory dwelling must be permanently attached to a foundation, or within a structure that is permanently attached to a foundation. - Parking. - No additional parking space is required for the accessory dwelling if it is created on a site with an existing house and on-street parking is permitted and adequate. - One additional parking space located on or within 100 feet of the lot is required for the accessory dwelling: 1) when none of the roadways in abutting streets can accommodate on-street parking; or 2) when the accessory dwelling unit is created at the same time as the principal dwelling. - An accessory dwelling is allowed on an existing lot of record with a nonconforming lot size and/or a non-conforming setback if the accessory dwelling does not increase the non-conforming setback. |  |
| --- | --- |

# ADU Ordinance - Required Provisions

| Required Provisions Required Provisions | Allowed By Right | • Allow Accessory Dwellings by right within any zone that permits single-unit detached residential development Comment: ADU’s must be allowed on lots with detached single-family dwelling units. Some jurisdictions may want to consider allowing ADU’s with attached (townhome), and duplex units, where adequate vehicular access and parking can be provided. |
| --- | --- | --- |
|  | No Density or Growth Restrictions | • A local jurisdiction may not regulate or restrict ADU’s using: - Residential density restrictions. (For example, an accessory dwelling on a lot with a primary dwelling unit, shall not count as a dwelling unit for purposes of calculating housing density.) - Residential growth limitation measures Comment: An accessory dwelling is treated as an allowable accessory use or structure, associated with the primary residential structure. |
|  | Must Comply with Safety & Building Codes | • Must provide for construction of ADUs that meet public health, safety, and welfare standards, including relevant building codes and adequate public facilities provisions. |
|  | Setbacks Same As Accessory Use Setbacks | • May not require side or rear yard setbacks that are greater than the side and rear yard setbacks for accessory structures in the applicable zoning district. Comment: In determining setbacks for detached ADUs, many jurisdictions have used five feet as the minimum separation between the ADU structure and the property line so that the accessory dwelling structure does not have to meet building code standard fire-resistance rating requirements. Also, a five-foot setback option allows for more windows and flexibility in window placement. |
|  | ADU Definition | An “Accessory Dwelling Unit” (ADU) is a secondary dwelling unit that is: - on the same lot, parcel, or tract as a primary single-family detached dwelling unit, and - has a gross floor area not greater than 75% of the size of the primary single-family detached dwelling unit. - is subordinate in use to the primary single-family detached dwelling unit. Comment: Definition is from MD 2024 HB 1466/SB 891 |

# ADU Ordinance - Optional Provisions

| Optional Provisions Optional Provisions | Owner Occupancy | Options: ✓ Owner-occupancy is not required for either the primary dwelling or the accessory dwelling (Recommended)  Primary dwelling must be owner-occupied  Either the primary dwelling or the accessory dwelling must be owner-occupied Comment: PRO’s: Owner-occupancy requirements are thought to ensure that property is better maintained and that the occupants are not disruptive to the neighborhood. An owner-occupancy requirement may be necessary to get local support. CON’s: An owner-occupancy requirement will reduce the number of accessory dwellings constructed, can be difficult to track and enforce, and may hinder resale and financing. Owner-occupancy requirements will deter some homeowners from constructing accessory dwellings because it will reduce the pool of potential future buyers. If the homeowners were to move, they would not have the option of leasing the ADU and their primary residence to separate tenants. New lending rules from the Federal Housing Administration allow mortgage borrowers to qualify in part based on income generated by renting out an ADU only if it can be rented without restriction.i Owner-occupancy compromise: Colorado had required owner occupancy only at the time of permitting. This provision aimed to deter investor speculation while protecting owners from being unable to rent their properties if circumstances change after permitting or construction. Note that Colorado recently eliminated any owner-occupancy requirement. |
| --- | --- | --- |
|  | Parking | Options: ✓ Additional parking is not required if the principal dwelling unit has sufficient off-street parking, or adequate additional onstreet parking is available within walking distance (500 feet) of the lot containing the accessory dwelling. (Recommended)  One off-street parking space is required for an ADU  One off-street parking space is required for each bedroom Comment: The cost and/or difficulty of complying with additional parking requirements for an ADU on an existing lot can often make the ADU cost prohibitive or physically impossible. Parking requirements should be as minimal as necessary to encourage ADU development. Recommend parking adequacy be staff-level review and approval. |

| Optional Provisions Optional Provisions | Minimum Floor Area |  | Options: ✓ No minimum floor area (Recommended)  Specify minimum floor area Comment: A minimum floor area can impose undue restrictions. A min. floor area of 800 sq. ft. would prevent a studio apartment above a 2-car garage, which could be as small as 400 sq. ft. Recommend no minimum floor area requirement, or alternatively a 400 sq. ft. min. floor area requirement. |
| --- | --- | --- | --- |
|  | Maximum Floor Area |  | Options: ✓ Not greater than 75% of the gross floor area of the primary dwelling unit (per State law) (Recommended)  A jurisdiction may set a maximum floor area that is less than 75%. For example, Carroll County requires that an ADU “shall have no more than 800 square feet of livable floor area or one-third of the total livable floor area of the principal dwelling unit, whichever is greater.” Comment: A maximum floor area ensures the ADU is truly a subordinate or accessory use to the primary home. |
|  | Bedrooms |  | Options: ✓ No more than 2 bedrooms are permitted in an ADU (Recommended)  Only 1 bedroom is permitted in an ADU Comment: The number of bedrooms permitted may vary depending on minimum or maximum floor area permitted. Most jurisdictions permit no more than 2 bedrooms, to ensure the ADU is truly a subordinate or accessory use to the primary home. Also, limiting to no more than 2 bedrooms is appropriate given the recommendation to waive other requirements, such as off-street parking, impact fees, APFO compliance, separate water and sewer taps. |
|  | Definitions for ADU Types |  |  Attached ADU  Internal ADU  Detached ADU Comment: These definitions may or may not be needed depending on other criteria imposed. For example, if setback requirements vary depending on whether an accessory dwelling is attached or detached, then definitions will be needed. |
| Optional Provisions Optional Provisions | Approval Process | ✓ An ADU that meets all zoning criteria, may be approved by staff without a public hearing. (Recommended) Comment: Allowing ADU’s to be approved by staff-level review is highly recommended. The zoning code should be drafted to provide clear guidelines for staff, but allow some discretion given the wide variety of site conditions. Staff review is preferable to a Special Exception of Conditional Use Permit hearing, during which emotions (rather than facts) can unduly influence the review and approval process. |  |
|  | Vehicular Access | Options: ✓ Not specify (Recommended)  Require vehicular access from a public street Comment: Recommend not specifying the vehicular access for the accessory dwelling, and allow staff-level review and approval of adequate vehicular access. The State law allows, but does not require, a local jurisdiction to prohibit conversion of an accessory structure to an accessory dwelling if the only vehicular access is an alley. |  |
|  | Access (Front Door) | Options: ✓ An ADU must have a direct exterior access that is separate from the existing primary dwelling. Administrator has discretion to waive if the entrance does not go into or through the primary dwelling unit (Recommended)  No requirement for direct outside access Comment: As the ADU is intended to be a complete independent living unit, recommend that ADU’s have a separate, direct exterior access. |  |
|  | Non- Conforming Lots | ✓ An accessory dwelling is allowed on an existing lot of record with a non-conforming lot size and/or a non-conforming setback if the accessory dwelling does not increase the non-conforming setback. (Recommended) Comment: This provision recognizes that many older, existing lots of record may not meet current zoning dimensional standards, and provides some flexibility to allow an ADU on a non-conforming lot. |  |
|  | Fire Sprinkler Systems | Comment: Do not recommend addressing sprinklers in ADU Ordinance. By State law, ADU’s are generally not required to have sprinkler systems if they are created within existing dwellings that do not have sprinkler systems. Maryland requires sprinkler systems to be installed in all newly constructed single-family and two-family dwellings, including ADU’s. Maryland Building Performance Standards Regulations, Md. Code Ann.§9.12.51.05(B)(1)(d). Recommend checking with local building code officials. |  |

| Optional Provisions Optional Provisions | APFO & Impact Fees | Options: ✓ An ADU does not have to comply with the APFO Ordinance  An ADU must comply with APFO requirements that are proportionate to the size of the ADU relative to the size of the primary dwelling unit. (For example, if an ADU has 500 sq. ft, and the primary unit has 2,000 sq. ft., the APFO requirements will be 25% of what is required for the primary dwelling unit)  An ADU must meet APFO requirements if constructed at the same time as the primary unit. Comment: It is important to keep in mind that ADU’s will only be constructed on a small percentage of lots. In Portland, Oregon where ADU’s have been permitted since 1981, only 2% of residential lots have an ADU.ii Where ADU’s are permitted, it is reported that additional impacts of ADU’s on public facilities have been minimal to non-existent. For example in Montgomery County, where ADU’s have been permitted since 2014, they have experienced no noticeable impact from ADU’s on school capacity.iii Alternatively, the AARP Model State Act and Local Ordinance recommends impact fees be waived for units less than 750 sq. ft., and charged proportionate to the size of the primary unit for ADUs over 750 sq. ft.iv Recognizing that ADU’s have a very small overall impact on public facilities, Anne Arundel County eliminated APFO requirements for ADU’s in 2023 per House Bill 6-23. |
| --- | --- | --- |
|  | Well and Septic | ✓ An ADU may share the well and septic of the primary dwelling unit, if adequate capacity exists |
|  | Public Water and Sewer | ✓ An Attached or Internal ADU must be serviced by the same public water and sewer lines as the principal dwelling unit. A Detached ADU may be serviced by the same public water and sewer lines as the principal dwelling unit where feasible, subject to approval by the local water or sewer authority. |
|  | Short-Term Rentals (rented for periods < 30 days) | Options: ✓ No short-term rental restrictions  An ADU may not be used as a Short-Term Rental  An ADU may only be used as a Short Term Rental if either the primary dwelling unit or the ADU is owner-occupied Comment: Recommend allowing ADU’s to be used for short-term rentals, particularly in tourist areas, and to help homeowners by providing supplementary income. However, if a jurisdiction is concerned about property maintenance, noise, or inadequate parking, it may want to prohibit use of ADU’s for short term rentals. |

# Examples of Existing ADU Ordinances in Maryland

NOTE:  These ADU Ordinance examples are provided to give local jurisdictions that have not previously allowed ADU’s a look at how other Maryland jurisdictions have regulated ADU’s in their zoning codes in the past, and the type of issues other jurisdictions have found to be relevant.  The 2025 State law took effect on October 1, 2025 and jurisdictions have until October 1, 2026 to comply with the new law.  These examples are NOT ordinances that have been drafted or revised to comply with the 2025 state law, and are for general reference only.

| Example Example | ANNAPOLIS, ANNE ARUNDEL COUNTY 21.64.010 - Accessory dwellings. - A maximum of one accessory dwelling unit may be established on any one lot. - An accessory dwelling unit may be established within a principal structure or within an accessory structure. - Each accessory dwelling unit shall have its own separate ingress and egress. - The addition of an impervious surface shall not be allowed on any lot containing an accessory dwelling unit, unless: - The Department has determined that the proposed surface will be treated or managed; or - The equivalent coverage of impervious surface will be removed elsewhere on the property. - The maximum size of any accessory dwelling unit that is not established within the existing footprint of the principal structure is eight hundred fifty square feet of livable space. - A rental license for any accessory dwelling unit is required in accordance with Chapter 17.44. - An accessory dwelling unit and principal structure on any one lot shall be in common ownership and the owner shall reside in either the accessory dwelling unit or the principal structure. (Ord. No. O-9-21, § I, 10-11-2021) |
| --- | --- |

| Example Example | CARROLL COUNTY: - Attached accessory dwelling units which are subject to the following: - An attached accessory dwelling must have direct access from the outside; - Only one attached accessory dwelling is permitted on any principal dwelling unit; - The property owner must occupy either the principal dwelling unit or the attached accessory dwelling unit on the property; - The maximum size of an attached accessory dwelling shall be 800 square feet of the livable floor area or one-third of the total livable floor area of the principal dwelling unit, whichever is greater; - The attached accessory dwelling unit shall have no more than 2 bedrooms; - The attached accessory dwelling unit must meet all applicable building construction and Maryland Department of Health and Mental Hygiene and MDE; - Two additional off street parking spaces must be provided for the attached accessory dwelling unit; and - Only one accessory dwelling unit (attached or detached) shall be permitted on a lot. - Detached accessory dwelling units, provided that the lot or parcel is eligible to be subdivided to separate the detached accessory dwelling and which are subject to the following: - Only one detached accessory dwelling unit is permitted on any lot or parcel. Buildings converted in accordance with § 158.070(D) shall be considered detached accessory dwelling units; - The property owner must occupy either the principal dwelling unit or the detached accessory dwelling unit on the lot or parcel; - The detached accessory dwelling unit must meet all applicable building construction and Maryland Department of Health and Mental Hygiene and MDE; - Two off street parking spaces must be provided for the detached accessory dwelling unit; and - Detached accessory dwelling units shall not be subject to any size limits. |
| --- | --- |

| Example Example | FREDERICK COUNTY: § 1-19-8.212 LIMITED ACCESSORY DWELLING UNITS IN THE RC, A, R1, R3, R5, R8, R12, R16, VC, MXD, PUD, AND MX DISTRICTS. The following provisions shall apply to limited accessory dwelling units in the RC, A, R1, R3, R5, R8, R12, R16, VC, MXD, PUD, and MX districts. - Only 1 limited accessory dwelling unit ("ADU") may be created on a lot. - A limited accessory dwelling unit shall be allowed within single-family dwellings, in an accessory structure, or built as a separate accessory structure on a single-family lot. - The owner of the property must reside in the principal dwelling or in the accessory dwelling unit. - There must be at least 1 additional parking space provided for the limited accessory dwelling unit. On-street parking may be utilized to meet this requirement. - The limited accessory dwelling unit shall not exceed 1,000 square feet in size, excluding decks, porches, patios or other items that are not part of the primary footprint of the accessory dwelling unit. An accessory dwelling unit greater than 1,000 square feet shall be reviewed as a special exception under § 1-19-8.321. - A limited accessory dwelling unit located in an accessory structure or built as a separate accessory structure must comply with the accessory structure requirements of § 1-198.240(B). - ADUs are intended to serve ongoing housing needs of county residents. Short term rental of ADUs in the nature of extended stay hotels, Airbnbs, or seasonal temporary housing is not permitted. - The owner of the principal residence shall file an annual statement with the Zoning Administrator verifying that conditions under which the limited accessory dwelling unit was granted remain the same. - If the ownership of the lot changes, the subsequent owner must provide a statement as to the continued use and eligibility of the accessory dwelling unit. - If the Zoning Administrator, after consultation with appropriate County staff, determines that the limited accessory dwelling unit is not in compliance with the above provisions as well as all safety, health, and environmental standards, approval of the accessory dwelling unit may be revoked pursuant to § 1-19-2.210. (Ord. 11-25-591, 10-27-2011; Ord. 11-28-594, 11-22-2011; Ord. 14-23-678, 11-13-2014; Bill No. 18-16, 8-21-2018; Bill No. 20-10, 7-21-2020) |
| --- | --- |

# ADU Ordinance – Other Resources

| Other Resources Other Resources | • • • • • | The ABCs of ADUs, A guide to Accessory Dwelling Units and how they expand housing options for people of all ages, AARP, 2021 (containing many photo examples of ADU’s) Accessory Dwelling Unit Policy Task Force Final Report, May 31, 2024, Maryland Department of Planning. Accessory Dwelling Units, Model State Act and Local Ordinance, ©2020-2021 by AARP, available at AARP.org/ADUs Expanding ADU Development and Occupancy: Solutions for Removing Local Barriers to ADU Construction, ©2023 by AARP. Inventory-of-ADU-Ordinances-For Website.pdf, Maryland Department of Planning. |
| --- | --- | --- |
|  | • | A Taxonomy of State Accessory Dwelling Unit Laws 2025 ¦ Mercatus Center, George Washington University, Emily Hamilton and Kol Peterson, Aug. 2025. |

i A Taxonomy of State Accessory Dwelling Unit Laws 2025 | Mercatus Center, George Washington University, Emily Hamilton and Kol Peterson, Aug. 2025. ii The ABCs of ADUs, A guide to Accessory Dwelling Units and how they expand housing options for people of all ages, AARP, 2021 iii Accessory Dwelling Unit Policy Task Force, Final Report, May 31, 2024, Maryland Department of Planning, page 19.

iv AARP – Accessory Dwelling Units:  Model State Act and Local Ordinance, page 19.